BOLETÍN INFORMATIVO DE LA

COMISIÓN DE GEOSPELEOLOGÍA

Federación Espeleológica de América Latina y el Caribe -FEALC-

- BICOMGEO - FEALC -

[Geospeleology Commission Newsletter,

Speleological Federation of Latin America and the Caribbean -FEALC-]

No. 42,  Diciembre  2003

Coordinador: Prof. Dr. Franco Urbani

Sociedad Venezolana de Espeleología. Apartado 47.334, Caracas 1041A, Venezuela.

Telefax: (58)-212-272-0724, Correo-e:   urbani@cantv.net

Este Boletín es de carácter informal -no arbitrado- preparado con el objetivo de divulgar rápidamente las actividades geoespeleológicas en la región de la FEALC. Sólo se difunde por vía de correo electrónico. Es de libre copia y difusión y explícitamente se solicita a quienes lo reciban que a su vez lo reenvíen a otros posibles interesados, o lo incluyan es páginas web. Todos los números anteriores están disponibles. Igualmente se pide que obtengan copias en papel para las bibliotecas de sus instituciones. Se solicitan contribuciones de cualquier tipo y extensión para su divulgación.

Índice

Cave Symbols - The Official UIS list																	 3-45

A proposition for Karst surface conventional symbols		      								45-49

Copied from  http://www.karto.ethz.ch/neumann-cgi/cave_symbol.pl

## Cave Symbols - The official UIS list

#### UIS working group on Cave Survey and Mapping

### Symbol: Survey Points. #### Image Plan; #### Image Section. #### Explanation The main survey points have been kept in the list, but often they are no longer shown on fair copies. #### Translations Croatian: Mjerne tocke Dutch: Meetpunten French: Point de relevé principal German: Messpunkte Italian: Caposaldo principale Romanian: Punct topografic principal Slovene: Merilne tocke.

### Symbol: Outline of a Gallery. #### Image Plan; #### Image Section. #### Explanation as before #### Translations Croatian: Stijene kanala/ galerije Dutch: gang-omlijning French: Dessin de galerie German: Gangumriss Italian: Contorno della cavitr Romanian: Conturul gaeriei Slovene: Stene rova.

### Symbol: Underlying Passage. #### Image Plan; #### Image Section. #### Explanation As an additional help to distinguish the lower from the upper passage it is recommended to stop the line of the lower gallery before it reaches the upper gallery. To distinguish this symbol from the "presumed dimensions of space" (see below) it is recommended to draw short dashes with wide spacing. #### Translations Croatian: Kanal na nizoj etazi Dutch: Onderliggende gang French: Croisement de galeries German: Unterlagernde Gänge Italian: Cavitr sottoposta Romanian: Galerii suprapuse Slovene: Spodaj lezeci rov.

### Symbol: Too narrow Continuation. #### Image Plan; #### Image Section. #### Explanation as before #### Translations Croatian: Preuski prolaz Dutch: te nauwe doorgang French: Continuations trop étroites German: Fortsetzung zu eng Italian: Prosecuzione troppo stretta Romanian: Impenetrabil Slovene: Preozko nadaljevanje.

### Symbol: continuation Possible. #### Image Plan; #### Image Section. #### Explanation as before #### Translations Croatian: Moguci nastavak Dutch: mogelijke doorgang French: Continuation possible German: Fortsetzung möglich Italian: Prosecuzione possibile Romanian: Continuare posibila Slovene: Mozno nadaljevanje.

### Symbol: Presumed dimensions of space. #### Image Plan; #### Image Section. #### Explanation as before. The dashes should be long and the spacing narrow (see "underlying passages"). #### Translations Croatian: Pretpostavljena velicina kanala/ galerije Dutch: Geschatte maten van de ruimte French: Dimensions spatiales présumées German: Geschätzte Gangdimensionen Italian: Dimensioni presunte della cavitr Romanian: Dimensiuni spatiale presupuse Slovene: Predvidena veliksot prostora.

### Symbol: Ceiling form. #### Image Plan; #### Image Section. #### Explanation The symbol for the form of the ceiling of a cave is also used for the dripline and for chimneys. #### Translations Croatian: Oblik stropa Dutch: vorm van het plafond French: Forme du plafond German: Deckenform Italian: Morfologia del soffitto Romanian: Forma tavanului Slovene: Oblika stropa.

### Symbol: Dripline - Profile. #### Image Plan; #### Image Section. #### Explanation The dripline can give valuable information especially for smaller caves and overhangs. It describes the actual entrance to a cave including the upper ceiling region and should therefore not be omitted. At the dripline the cave begins. Cross-sections are of major importance for the understanding of a passage. For each map passage cross-sections must be drawn. The arrows indicating the direction of view are done differently, a clear indication is, however, necessary. #### Translations Croatian: kapnica (pocetak spilje) - Presjek Dutch: druplijn - profiel French: Entrée de cavité - Sections transversales German: Trauflinie - Querschnitt Italian: Limite esterno della grotta - Sezione trasversal Romanian: Intrare-sectiune transversala Slovene: Zunanja meja jame - profil.

### Symbol: Steps. #### Image Plan; #### Image Section. #### Explanation This symbol is well known, just don't forget that the little dashes are always to be orientated downwards! #### Translations Croatian: Skok Dutch: treden French: Marches, ressauts German: Stufen Italian: Salto Romanian: Saritoare Slovene: Stopnja.

### Symbol: Pit. #### Image Plan; #### Image Section. #### Explanation The filled or empty triangle have been replaced by dashed lines for two reasons: First, the dashed line is easier to draw, secondly it is unclear even to most draughtsmen whether the empty triangles represent the smaller or the higher steps or when they should be filled (at 5 m or 10 m?). #### Translations Croatian: Vertikalni kanal (jama u objektu) Dutch: Put/schacht French: Puits (profondeur en mctres) German: Schacht Italian: Pozzo Romanian: Put (adâncime în metri) Slovene: Brezno.

### Symbol: Pit open to Surface. #### Image Plan; #### Image Section. #### Explanation as before #### Translations Croatian: Vertikalni ulaz (otvor jame) Dutch: Put, Daglichtput French: Puits débouchant en surface German: Schacht mit Oberflächenöffnung Italian: Pozzo 2 Romanian: Aven Slovene: Vhodno brezno.

### Symbol: Aven/Aven-pitchs. #### Image Plan; #### Image Section. #### Explanation The plus and minus signs can be written inside or outside the cave. For reasons of clearness they should be encircled when written outside the cave. We agreed to the following procedure to indicate the difficulty of a drop or a pitch: If the drop can be climbed without gear, there is only the number (in meters) written. If gear is needed, any letter precedes the number. This way, the use of most countries is respected. It is optional to draw the anchor points and ropes into the longitudinal section. #### Translations Croatian: Dimnjak/ Dimnjak iznad jame Dutch: schoorsteen - put French: Cheminée-Puits-cheminée German: Schacht - Schlot Italian: Camino - Pozzo-Camino Romanian: Horn - Horn-put Slovene: Kamin - brezno s kaminom.

### Symbol: Contourlines. #### Image Plan. #### Explanation Contour Lines (lines of same altitude) with altimetric measurement as done on geographic maps are seldom used, as the exact ground level is not always easily established and precise drawings can cause serious problems. Gradient lines are used more frequently. #### Translations Croatian: Slojnice sa oznacenom nadmorskom visinom Dutch: Contourlijnen French: Courbes de niveau German: Höhenkurven Italian: Curve di livello Romanian: Curbe de nivel (altitudine în metri) Slovene: Izohipse.

### Symbol: Gradient Arrows - Entrance Arrow. #### Image Plan. #### Explanation The differentiation between gradient arrows inside and outside the cave is now omitted. An arrow indicating the entrance was added to the list. #### Translations Croatian: Smjer nagiba - Ulazna strelica Dutch: Daal-pijlen Ingangs-pijl French: Flcche de gradient - Flcche indiquant l'entrée de la cavité German: Gefällepfeile - Eingangspfeil Italian: Freccia di gradiente - Freccia per l'ingresso Romanian: Directia de panta-Saggeata indicând intrarea Slovene: Smer naklona - puscica za vhod.

### Symbol: Gradient Lines. #### Image Plan. #### Explanation Gradient lines are the most common methods to illustrate the ground structure. As on geographic maps narrow spacing indicates a steep and wide spacing a gentle slope. #### Translations Croatian: Slojnice (nagibne linije) Dutch: Overgangslijnen French: Lignes de gradient German: Gefällelinien Italian: Linee di Gradiente Romanian: Linie de panta Slovene: Naklonske crte.

### Symbol: Altitude above Sealevel. #### Image Plan. #### Explanation This symbol has also been kept for reasons of consistence and designates the altitude above sea-level used in the map. It is highly recommended to write 1880 m. instead of just writing 1880. Generally maps are all to be done in SI units. #### Translations Croatian: Nadmorska visina (m.n.m) Dutch: Hoogte boven zeeniveau French: Altitude au-dessus du niveau de la mer German: Höhenangaben (Seehöhe) Italian: Quota s.l.m. Romanian: Altitudini deasupra nivelului marii Slovene: Nadmorska visina (m.n.m.).

### Symbol: Difference in Elevation. #### Image Plan. #### Explanation in Relation to the entrance, as before #### Translations Croatian: Visinska razlika Dutch: Hoogteverschil French: Altitude relative r l'entrée de la cavité German: Höhendifferenz Italian: Differenza di quota Romanian: Altitudine relativa de la intrare Slovene: Visinska razlika.

### Symbol: Joint - Fault - Bedding Plane - Inclined Joint. #### Image Plan. #### Explanation These symbols can be quite important for the understanding of the cave. Please use the symbols proposed here only when there's no appropriate set of symbols at the geological survey of your country. It is also recommended not to use any geological symbol unless one is totally sure about its nature. The arrow connecting two joint symbols is normally only used in the plan. It shows whether a cave crosses the same joint several times or whether it follows the joint itself. #### Translations Croatian: Pukotina - Rasjed - Geoloski sloj - Nagnuta pukotina Dutch: Breuk - Verschuiving - Laagvoeg French: Fracture - faille - joint de strate - fracture inclince German: Kluft - Verwerfung - Schichtfuge - schräge Ebene Italian: Frattura-Faglia-Stratificazione Romanian: Fractura-Falie-Fata de strat-Fractura înclinata Slovene: Razpoka - prelom - geoloska plast - nagnjena razpoka.

### Symbol: Lake - Flowing Water. #### Image Plan; #### Image Section. #### Explanation as before #### Translations Croatian: Jezero - Vodeni tok Dutch: Meer - Stromend water French: Lac-eau courante German: Seen und fliessende Gewässer Italian: Lago-Corso d'acqua Romanian: Lac - apa curgatoare Slovene: Jezero - vodni tok.

### Symbol: Sump. #### Image Plan; #### Image Section. #### Explanation as before. As an option it is possible to stop the crosshatching in order to draw the characteristics of the sump. This possibility, especially useful in longer sumps, is also shown. #### Translations Croatian: Sifon Dutch: sifon/watergevulde gang French: Siphon German: Siphon Italian: Sifone Romanian: Sifon Slovene: Sifon.

### Symbol: Cascade - Waterfall. #### Image Plan; #### Image Section. #### Explanation The symbol for waterfall is the same as the one for drop. To show flowing water one can draw an arrow indicating the flow before or at the side. #### Translations Croatian: Vodopad Dutch: Cascade - Waterval French: Cascade / Chute d'eau German: Wasserfall Italian: Cascata Romanian: missing Slovene: Slap.

### Symbol: Spring - Sink. #### Image Plan; #### Image Section. #### Explanation Springs or sinks which flow sideways in or out of a cave don't need a half circle to be characterised, whereas water inlets and outlets in the ground should be marked with a half-circle to distinguish them clearly from normal flowing water. Symbols for temporary waters of all kinds have been omitted alltogether because of the following reasons: Hydrologically many water courses are also dry in dry times and are therefore also temporary. Adding to that, there have been many different symbols in the old symbol lists that would severely extend this list. One may add a note on temporary waters to the cave description. #### Translations Croatian: Izvor - Ponor Dutch: Bron - verdwijnpunt French: Source / perte German: Quelle - Ponor Italian: Sorgente - Inghiottitoio Romanian: Izvor - Ponor-Pierdere de apa Slovene: Izvir - ponor.

### Symbol: Widespread Water Inlet - Seeping. #### Image Plan; #### Image Section. #### Explanation There is no special symbol any more. It is needed to draw several ± parallel arrows indicating the flow. For seeping of a watercourse in sediment the semicircle is also used in the plan! In the longitudinal section the direction of the arrow can be bent towards the bottom to hint the seeping. #### Translations Croatian: Razveden dotok vode - Sitasti ponor Dutch: wijdverspreide waterinvoer - sijpels French: Arrivée d'eau diffuse - Suintement d'eau dans un sédiment German: Tropfwasser - Sickerwasser Italian: Afflusso d'acqua diffuso - Infiltrazione nel detrito Romanian: Aport de apa difuz - Infiltratii Slovene: Razvejan pritok vode - ponikujoca voda.

### Symbol: Scallops, Flutes in General, Direction of Paleoflow. #### Image Plan; #### Image Section. #### Explanation Flow Casts (Scallops): as before. Flute: The differentiation between erosional and corrosion flutes has been omitted, because, especially with flutes on lateral walls, they cannot always be identified correctly. The new symbol can be rotated; if the semicircle is on top, it shows a flute on the ceiling, etc. Paleoflow: This very useful symbol is not only used with scallops, but also with ripple marks, imbrications and other features. In addition it might be used in lava caves to show the flow direction of the lava. #### Translations Croatian: Vrtlozni lonci - Zljebici (fasete) - Smjer fosilnog toka Dutch: Erosie koepels, kolkgaten, richting van de paleo-stroom French: Coupoles d'érosion - marmites - courant d'ccoulement fossile German: Kolke - Fliessfacetten - Paläofliessrichtung Italian: Scallops - Scanellature in general - Direzione del paleoflusso Romanian: Cupole de eroziune - marmite - curent de apa fosil Slovene: Kotlice - zlebici (fasete) - smer starega toka..

### Symbol: Air draught - Ice/Snow/Firn. #### Image Plan; #### Image Section. #### Explanation Air draught: This is a well known symbol. It should always include the date of measurement. The symbol now uses always two feathers! The date of measurement should be written in the standard dd-mm-yyyy format! Ice/Snow/Firn: The old star symbol used for snow hasn´t been satisfactory for quite some time as it is very time consuming to draw and the ground structure cannot be shown. Added to this, it can easily be mistaken for the crystal symbol. The old symbol for ice is confusingly similar to true gradient lines. Taking all this into consideration we agreed on the following solution: Snow is transformed to firn and subsequently to ice. By morphology and composition all three substances are the same, therefore it should be represented by the same symbol. The one that has been chosen is easy to draw, shows the ground structure and cannot be mixed up. #### Translations Croatian: Propuh - Led/Snijeg/Inje Dutch: luchtstroom - Ijs-Sneeuw-firn French: Courant d'air - Glace/neige/givre German: Luftzug - Eis/Schnee/Firn Italian: Corrente d'aria - Ghiaccio - Neve - Firn Romanian: Curent de aer - Gheata-zapada-firn Slovene: Prepih - led/sneg/srez.

### Symbol: Stalagmites. #### Image Plan; #### Image Section. #### Explanation as before #### Translations Croatian: Stalagmiti Dutch: Stalagmieten French: Stalagmites German: Stalagmiten Italian: Stalagmiti Romanian: Stalagmite Slovene: Stalagmiti.

### Symbol: Stalactites. #### Image Plan; #### Image Section. #### Explanation as before #### Translations Croatian: Stalaktiti Dutch: Stalactieten French: Stalactites German: Stalaktiten Italian: Stalattiti Romanian: Stalactite Slovene: Stalaktiti.

### Symbol: Sinter Curtains - Pillars. #### Image Plan; #### Image Section. #### Explanation Calcite Curtains: The presented solution is not easily understood at first sight but seems to be the best (alternatively you draw the curtains into your map). Calcite pillars: as before #### Translations Croatian: Zavjese od sigovine - Stupovi Dutch: Gordijnen - Pilaren French: Draperies stalagmitiques - Piliers stalagmitiques German: Sintervorhänge - Tropfsteinsäulen Italian: Cortina di concrezione - Colonna Romanian: Draperii stalagmitice - coloane-pilieri stalagmitici Slovene: Sigova zavesa - steber.

### Symbol: Helictites - Soda Straws - Crystals. #### Image Plan; #### Image Section. #### Explanation Helictites: This symbol is as eccentric as the formation itself and is therefore easily understood. Soda Straws: This symbol doesn´t need any further explanations. Crystals: For reasons of international understanding the letters identifying the minerals should be avoided, first because correct mineral identification is often not as easy as it may seem, and second because a C for calcite is an K in German and a # in Chinese. In general we do not recommend the use of letters as symbols on the actual map. The resulting babylonic confusion can be lethal for the understanding. #### Translations Croatian: Heliktiti - Spageti - Kristali Dutch: excentrieken - macaronies - kristallen French: Excentriques/Helictites - Spaghettis - Cristaux German: Excentriques - Spaghetti - Kristalle Italian: Concrezione eccentrica - Spaghetti - Cristalli Romanian: Excentrite-Helictite - macaroane - cristale Slovene: Heliktiti - spageti (cevcice) - kristali.

### Symbol: Sinter Pools. #### Image Plan; #### Image Section. #### Explanation as before #### Translations Croatian: Zasigane kaskade Dutch: Sinter-bekkens French: Gours German: Sinterbecken Italian: Grandi vasche - Vaschette di concrezione Romanian: Gururi Slovene: Sigove ponvice.

### Symbol: Flowstone - Wall Calcite - Moonmilk. #### Image Plan; #### Image Section. #### Explanation Flowstone: Often this or a similar symbol is also used for clay (see there). Wall Calcite: as before Moonmilk: To avoid mistaking this symbol for the guano symbol an additional bow has been added to the usual m. #### Translations Croatian: Zasigano tlo - Zasigana stijena - Spiljsko mlijeko Dutch: Bodemdruipsteen - afzettingen op de wand - bergmelk French: Concrétionnement au sol - Concrétionnement sur les parois - Mondmilch German: Bodensinter - Wandsinter - Mondmilch (Bergmilch) Italian: Colata a pavimento - Colata parietale - Latte di Monte Romanian: Planseu concretionat - concretiuni parietale - montmilch Slovene: Zasigano pobocje - zasigana stena - jamsko mleko.

### Symbol: North Arrow Geographic and Cartesic. #### Image Plan. #### Explanation On several maps, information on declination is missing or is incomplete, which makes the calculation of geographic north almost impossible. Beside the normal North arrow one must specify whether one relates to geographic, magnetic, or cartographers north. If one is relating to magnetic north the date of measurement must be mentioned. Both versions shown are equivalent. #### Translations Croatian: Zemljopisni (geografski) i Kartezijski sjever Dutch: Noord pijl Geografisch en Cartesisch French: Nord géographique et cartographique German: Geographisch und Kartesisch Nord (Gitternord) Italian: Nord Geografico et Cartesiano Romanian: Directia nord - geografic and cartigrafic Slovene: Geografski sever - Kartezijski sever.

### Symbol: North Arrow Magnetic. #### Image Plan. #### Explanation On several maps, information on declination is missing or is incomplete, which makes the calculation of geographic north almost impossible. Beside the normal North arrow one must specify whether one relates to geographic, magnetic, or cartographers north. If one is relating to magnetic north the date of measurement must be mentioned. #### Translations Croatian: Magnetski sjever Dutch: Noordpijl Magnetisch Noorden French: Nord magnétique German: Magnetisch Nord Italian: Nord Magnetico Romanian: Directia nord magnetic Slovene: Magnetni sever.

### Symbol: Blocks - Debris. #### Image Plan; #### Image Section. #### Explanation as before. The size can be modelled on the reality. #### Translations Croatian: Blokovi - Krslje Dutch: Blokken - rol-steentjes French: Blocs, débris German: Blockwerk, Geröll Italian: Massi - Detriti a spigoli vivi Romanian: Blocuri - prabusiri Slovene: Bloki - grusc.

### Symbol: Pebbles. #### Image Plan; #### Image Section. #### Explanation as before. The size can be modelled on the reality. #### Translations Croatian: Sljunak Dutch: Kiezels French: Cailloux German: Kiesel Italian: Ciottoli - Detriti a spigoli arrotondati Romanian: Pietris Slovene: Prodniki.

### Symbol: Clastic sediments: Sand - Silt - Clay - Humus. #### Image Plan; #### Image Section. #### Explanation With the exception of humus this group is a sediment whose grain size and therefore names depend on the velocity of current of the carrying water. In flowing water sand is deposited whereas in standing water clay is deposited. In contrast to this the flowstone (see there), which is a chemical precipitchation, does not represent the velocity of flow but the chemical equilibrium. As flowstone occurs quite frequently a special symbol needed to be found. The small parallel dashes did a good job as they also showed the ground structure. Unfortunately several surveyors characterised clay by a series of small dashes, which can lead to some confusion. #### Translations Croatian: Klasticni sedimenti: Pijesak - Mulj - Glina - Humus Dutch: Sedimenten: Zand - Zilt - Klei/Leem - Humus French: Sédiments clastiques : Sable, limon, argile et humus German: Klastische Sedimente: Sand - Schlamm - Lehm - Erde Italian: Sedimento fine: Sabbia - Limo - Argilla - Humus Romanian: Sediment clastic - nisip, mâl, argila, humus Slovene: Klasticni sedimenti: Pesek - mulj - glina (blato) - prst.

### Symbol: Clay-Covered Walls. #### Image Plan; #### Image Section. #### Explanation This symbol can be used to show flooded zones. To show a mud coating, one thin dotted line is drawn behind the cave walls. #### Translations Croatian: Glinom prekrivene stijene Dutch: Klei-bedekte wanden French: Parois couvertes d'argile German: Lehmüberzogene Wände Italian: Pareti ricoperte di argilla Romanian: Pereti acoperiti cu argila Slovene: Z glino prekrite stene.

### Symbol: Guano. #### Image Plan; #### Image Section. #### Explanation as before #### Translations Croatian: Guano Dutch: Guano French: Guano German: Guano Italian: Guano Romanian: Guano Slovene: guano.

### Symbol: Camp. #### Image Plan; #### Image Section. #### Explanation as before #### Translations Croatian: Bivak Dutch: Kamp/Bivak French: Camp German: Biwak Italian: Campo, Bivacco Romanian: Tabara Slovene: bivak.

### Symbol: Height of a Room. #### Image Plan. #### Explanation An indication on height can be useful for a caver who doesn´t draw longitudinal sections. But since this is highly recommended (see documentation of the 9. National Nongress 991, Charmey) this symbol is not going to be used very often. #### Translations Croatian: Visina galerije Dutch: Hoogte van een zaal French: Hauteur d'une galerie / salle German: Raumhöhe Italian: Altezza di un ambiente Romanian: Înaltime (alerie-sala) Slovene: Visina prostora.

### Symbol: Anostomosen - Karren. #### Image Plan; #### Image Section. #### Explanation as before #### Translations Croatian: Anastomoza - Skrape Dutch: Vlechtende micro-gangetjes - Karren French: Anastomoses - Lapiez German: Anostomosen - Karren Italian: Anastomosi - Karren Romanian: Anastomoze - lapiezuri Slovene: Anastomoza - skraplje.

### Symbol: Cauliflower-Calcite / Disk. #### Image Plan; #### Image Section. #### Explanation These two symbols for calcite are of a regional importance and can be quite useful. #### Translations Croatian: Karfioli - Nakapne ploce Dutch: Kalk-bloemkool - Disk French: Concrétionnement en choux-fleurs - Disques German: Blumenkohlsinter / Disk Italian: Cavolfiori - Dischi di concrezione Romanian: Clusterite-Coralite - discuri Slovene: Karfjolce - kapniske plosce.

### Symbol: Bones. #### Image Plan; #### Image Section. #### Explanation This symbol does not need any further explanation. #### Translations Croatian: Kosti Dutch: Botten French: Ossements German: Knochen Italian: Ossa Romanian: Oase Slovene: kosti.

### Symbol: Human Activity. #### Image Plan; #### Image Section. #### Explanation This symbol is used for man-made changements and/or relicts in caves, including e.g. artwork / drawing / pottery / old mining sites / human bones. Preferably, it will be used for historic/prehistoric changements! #### Translations Croatian: Ljudska aktivnost Dutch: Menselijke activiteit French: Traces d'activité humaine German: Menschliche Artefakte Italian: Modificazioni umane Romanian: Urme de activitate antropica Slovene: Clovekove spremembe oz. ostanki.

Tomado de: http://www.earthsci.unibe.ch/people/praezis/symbol2.html

http://www.earthsci.unibe.ch/people/praezis/symbol1.html

A proposition for Karst surface conventional symbols

Karst surface symbols, used mainly by geomorphologists, but sometimes also by geologists, contain important information on a map. Until now, however, the surface symbols were only partly standardised, often on a national (or even organisational) scale. This led to a possible misunderstanding of the map or to a difficult reading that needed to consult the legend very often. The utility to standardize the karst surface symbols is therefore given.

The following propositions had been elaborated on the base of several regional or national lists, most of them coming from Europe. Two of the main contributors are the work of F. JOLY  and A. BINI et al., that were completed and partially adapted to the Swiss Geologic conventional signs, and the old UIS entrance symbol set. The present proposition's aim is:

- to be a base of information. This means that the present list is by no means complete, but should constitute a sound base, upon which the individual countries and/or organisations can build up their own symbol list they need for their specific maps

- therefore to stimulate discussions amongst karst geomorphologists and geologists

- to get to a generally adopted, universally acknowledged symbol set that will facititate the reading of geomorphologic maps.

It therefore follows the UIS Cave Signatures which had been elaborated on the same way.

The present proposition had been elaborated by the UISIC Working group Survey and Mapping, in contact with the IGU Karst Commission and other interested persons. Attention has been paid that the presented symbols can be generated or propagated by computer programs, since most of today's cartography is computer-aided.

The author of the present list kindly asks for suggestions, improvements, and comments on the present proposition!

The address is:

Ph. Haeuselmann   Haldenstr. 32, 3014 Bern - Switzerland,  0041 31 3320174,  praezis@geo.unibe.ch